Міністерство освіти і науки України

Національний університет «Львівська політехніка»

Кафедра ЕОМ

Звіт

до лабораторної роботи № 3

з дисципліни «Моделювання комп’ютерних систем»

на тему:

«Поведінковий опис цифрового автомата Перевірка роботи автомата за допомогою стенда Elbert V2 – Spartan 3A FPGA»

Варіант №4

Виконав:

ст. гр. КІ-201

Бойко Т. О.

Прийняв:

ст. викладач 
каф. ЕОМ
Козак Н. Б.

Львів 2024

Мета роботи: На базі стенда реалізувати цифровий автомат для обчислення значення виразів.

Виконання роботи:

Рис. 1 – Top Level

Файл ACC.vhd library IEEE; use IEEE.STD_LOGIC_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --use IEEE.NUMERIC_STD.ALL; -- Uncomment the following library declaration if instantiating -- any Xilinx primitives in this code. --library UNISIM; --use UNISIM.VComponents.all; entity ACC is Port ( WR : in STD_LOGIC; RESET : in STD_LOGIC; CLK : in STD_LOGIC; INPUT : in STD_LOGIC_VECTOR (7 downto 0); OUTPUT : out STD_LOGIC_VECTOR (7 downto 0)); end ACC; architecture ACC_arch of ACC is signal DATA : STD_LOGIC_VECTOR (7 downto 0); begin process (CLK) begin if rising_edge(CLK) then if RESET = '1' then DATA <= (others => '0'); elsif WR = '1' then DATA <= INPUT; end if; end if; end process; OUTPUT <= DATA; end ACC_arch;

Файл ALU.vhd library IEEE; use IEEE.STD_LOGIC_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values use IEEE.NUMERIC_STD.ALL; use IEEE.STD_LOGIC_UNSIGNED.ALL; -- Uncomment the following library declaration if instantiating -- any Xilinx primitives in this code. --library UNISIM; --use UNISIM.VComponents.all; entity ALU is Port ( A : in STD_LOGIC_VECTOR(7 downto 0); B : in STD_LOGIC_VECTOR(7 downto 0); OP : in STD_LOGIC_VECTOR(1 downto 0); OUTPUT : out STD_LOGIC_VECTOR(7 downto 0); OVERFLOW: out STD_LOGIC); end ALU; architecture ALU_Behavioral of ALU is component FullAdder8 is Port ( A: in STD_LOGIC_VECTOR (7 downto 0); B: in STD_LOGIC_VECTOR (7 downto 0); Ci: in STD_LOGIC; S: out STD_LOGIC_VECTOR (7 downto 0); Co: out STD_LOGIC); end component; signal AdderOut: STD_LOGIC_VECTOR(7 downto 0) := (others => '0'); signal Carry: STD_LOGIC; signal SEL: STD_LOGIC := '0'; signal MuxB: STD_LOGIC_VECTOR(7 downto 0) := (others => '0'); begin MuxB <= B when (SEL = '0') else not B; Adder: FullAdder8 port map( A => A, B => MuxB, Ci => SEL, S => AdderOut, Co => Carry ); ALU_EX: process (A, B, OP, Carry, AdderOut) begin case (OP) is when "00" => OUTPUT <= A xor B; OVERFLOW <= '0'; when "01" => SEL <= '0'; OUTPUT <= AdderOut; OVERFLOW <= Carry; when "10" => SEL <= '1'; OUTPUT <= AdderOut; OVERFLOW <= not Carry; when others => OUTPUT <= (others => '0'); OVERFLOW <= '1'; end case; end process; end ALU_Behavioral;

Файл CPU.vhd library IEEE; use IEEE.STD_LOGIC_1164.ALL; entity CPU is port( ENTER_OP1 : IN STD_LOGIC; ENTER_OP2 : IN STD_LOGIC; CALCULATE : IN STD_LOGIC; RESET : IN STD_LOGIC; CLOCK : IN STD_LOGIC; RAM_WR : OUT STD_LOGIC; RAM_ADDR : OUT STD_LOGIC_VECTOR(1 DOWNTO 0); CONST : OUT STD_LOGIC_VECTOR(7 DOWNTO 0); ACC_WR : OUT STD_LOGIC; ACC_RST : OUT STD_LOGIC; IN_SEL : OUT STD_LOGIC_VECTOR(1 downto 0); OP : OUT STD_LOGIC_VECTOR(1 DOWNTO 0)); end CPU; architecture CPU_arch of CPU is type STATE_TYPE is (RST, IDLE, LOAD_OP1, LOAD_OP2, RUN_CALC0, RUN_CALC1, RUN_CALC2, RUN_CALC3, RUN_CALC4, FINISH); signal CUR_STATE : STATE_TYPE; signal NEXT_STATE : STATE_TYPE; begin SYNC_PROC: process (CLOCK) begin if (rising_edge(CLOCK)) then if (RESET = '1') then CUR_STATE <= RST; else CUR_STATE <= NEXT_STATE; end if; end if; end process; NEXT_STATE_DECODE: process (CLOCK, ENTER_OP1, ENTER_OP2, CALCULATE) begin NEXT_STATE <= CUR_STATE; case(CUR_STATE) is when RST => NEXT_STATE <= IDLE; when IDLE => if (ENTER_OP1 = '1') then NEXT_STATE <= LOAD_OP1; elsif (ENTER_OP2 = '1') then NEXT_STATE <= LOAD_OP2; elsif (CALCULATE = '1') then NEXT_STATE <= RUN_CALC0; else NEXT_STATE <= IDLE; end if; when LOAD_OP1 => NEXT_STATE <= IDLE; when LOAD_OP2 => NEXT_STATE <= IDLE; when RUN_CALC0 => NEXT_STATE <= RUN_CALC1; when RUN_CALC1 => NEXT_STATE <= RUN_CALC2; when RUN_CALC2 => NEXT_STATE <= RUN_CALC3; when RUN_CALC3 => NEXT_STATE <= RUN_CALC4; when RUN_CALC4 => NEXT_STATE <= FINISH; when FINISH => NEXT_STATE <= FINISH; when others => NEXT_STATE <= IDLE; end case; end process; OUTPUT_DECODE: process (CUR_STATE) begin case (CUR_STATE) is when RST => RAM_WR <= '0'; RAM_ADDR <= "00"; CONST <= "00000000"; ACC_WR <= '0'; ACC_RST <= '1'; IN_SEL <= "00"; OP <= "00"; when LOAD_OP1 => RAM_WR <= '1'; RAM_ADDR <= "00"; CONST <= "00000000"; ACC_WR <= '0'; ACC_RST <= '1'; IN_SEL <= "00"; OP <= "01"; when LOAD_OP2 => RAM_WR <= '1'; RAM_ADDR <= "01"; CONST <= "00000000"; ACC_WR <= '0'; ACC_RST <= '1'; IN_SEL <= "00"; OP <= "01"; when RUN_CALC0 => RAM_WR <= '0'; RAM_ADDR <= "00"; CONST <= "00000000"; ACC_WR <= '1'; ACC_RST <= '0'; IN_SEL <= "01"; OP <= "01"; when RUN_CALC1 => RAM_WR <= '0'; RAM_ADDR <= "01"; CONST <= "00000000"; ACC_WR <= '1'; ACC_RST <= '0'; IN_SEL <= "01"; OP <= "01"; when RUN_CALC2 => RAM_WR <= '0'; RAM_ADDR <= "01"; CONST <= "00001010"; ACC_WR <= '1'; ACC_RST <= '0'; IN_SEL <= "10"; OP <= "01"; when RUN_CALC3 => RAM_WR <= '0'; RAM_ADDR <= "01"; CONST <= "00000000"; ACC_WR <= '1'; ACC_RST <= '0'; IN_SEL <= "01"; OP <= "00"; when RUN_CALC4 => RAM_WR <= '0'; RAM_ADDR <= "00"; CONST <= "00000000"; ACC_WR <= '1'; ACC_RST <= '0'; IN_SEL <= "01"; OP <= "10"; when IDLE => RAM_WR <= '0'; RAM_ADDR <= "00"; CONST <= "00000000"; ACC_WR <= '0'; ACC_RST <= '0'; IN_SEL <= "00"; OP <= "00"; when others => RAM_WR <= '0'; RAM_ADDR <= "00"; CONST <= "00000000"; ACC_WR <= '0'; ACC_RST <= '0'; IN_SEL <= "00"; OP <= "00"; end case; end process; end CPU_arch;

Файл FullAdder.vhd library IEEE; use IEEE.STD_LOGIC_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --use IEEE.NUMERIC_STD.ALL; -- Uncomment the following library declaration if instantiating -- any Xilinx primitives in this code. --library UNISIM; --use UNISIM.VComponents.all; entity FullAdder is Port (A: in STD_LOGIC; B: in STD_LOGIC; Ci: in STD_LOGIC; S: out STD_LOGIC; Co: out STD_LOGIC); end FullAdder; architecture Behavioral of FullAdder is begin S <= (A xor B) xor Ci; Co <= (A and B) or ((A xor B) and Ci); end Behavioral;

Файл FullAdder8.vhd library IEEE; use IEEE.STD_LOGIC_1164.ALL; -- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --use IEEE.NUMERIC_STD.ALL; -- Uncomment the following library declaration if instantiating -- any Xilinx primitives in this code. --library UNISIM; --use UNISIM.VComponents.all; entity FullAdder8 is Port ( A: in STD_LOGIC_VECTOR (7 downto 0); B: in STD_LOGIC_VECTOR (7 downto 0); Ci: in STD_LOGIC; S: out STD_LOGIC_VECTOR (7 downto 0); Co: out STD_LOGIC); end FullAdder8; architecture Behavioral of FullAdder8 is component FullAdder is Port (A: in STD_LOGIC; B: in STD_LOGIC; Ci: in STD_LOGIC; S: out STD_LOGIC; Co: out STD_LOGIC); end component; signal carry: STD_LOGIC_VECTOR(8 downto 0) := (others => '0'); begin carry(0) <= Ci; FullAdderGenerate: for i in 0 to 7 generate adder: FullAdder port map( A => A(i), B => B(i), Ci => carry(i), S => S(i), Co => carry(i + 1) ); end generate FullAdderGenerate; Co <= carry(8); end Behavioral;

Файл MUX.vhd library IEEE; use IEEE.STD_LOGIC_1164.ALL; entity MUX is PORT( SEL: in STD_LOGIC_VECTOR(1 downto 0); CONST: in STD_LOGIC_VECTOR(7 downto 0); --CONST1: in STD_LOGIC_VECTOR() DATA_IN0: in STD_LOGIC_VECTOR(7 downto 0); DATA_IN1: in STD_LOGIC_VECTOR(7 downto 0); OUTPUT: out STD_LOGIC_VECTOR(7 downto 0) ); end MUX; architecture Behavioral of MUX is begin process (SEL, DATA_IN0, DATA_IN1, CONST) begin if (SEL = "00") then OUTPUT <= DATA_IN0; elsif (SEL = "01") then OUTPUT <= DATA_IN1; else OUTPUT <= CONST; end if; end process; end Behavioral;

Файл RAM.vhd library IEEE; use IEEE.STD_LOGIC_1164.ALL; use IEEE.NUMERIC_STD.ALL; use IEEE.STD_LOGIC_UNSIGNED.ALL; entity RAM is port( WR : IN STD_LOGIC; ADDR : IN STD_LOGIC_VECTOR(1 DOWNTO 0); DATA : IN STD_LOGIC_VECTOR(7 DOWNTO 0); CLOCK: IN STD_LOGIC; OUTPUT : OUT STD_LOGIC_VECTOR(7 DOWNTO 0) ); end RAM; architecture RAM_arch of RAM is type ram_type is array (3 downto 0) of STD_LOGIC_VECTOR(7 downto 0); signal UNIT : ram_type; begin process(ADDR, CLOCK, UNIT) begin if(rising_edge(CLOCK)) then if (WR = '1') then UNIT(conv_integer(ADDR)) <= DATA; end if; end if; OUTPUT <= UNIT(conv_integer(ADDR)); end process; end RAM_arch;

Файл SEG_DECODER.vhd library IEEE; use IEEE.STD_LOGIC_1164.ALL; use IEEE.STD_LOGIC_ARITH.ALL; use IEEE.STD_LOGIC_UNSIGNED.ALL; entity SEG_DECODER is port( CLOCK : IN STD_LOGIC; RESET : IN STD_LOGIC; ACC_DATA_OUT_BUS : IN STD_LOGIC_VECTOR(7 DOWNTO 0); COMM_ONES : OUT STD_LOGIC; COMM_DECS : OUT STD_LOGIC; COMM_HUNDREDS : OUT STD_LOGIC; SEG_A : OUT STD_LOGIC; SEG_B : OUT STD_LOGIC; SEG_C : OUT STD_LOGIC; SEG_D : OUT STD_LOGIC; SEG_E : OUT STD_LOGIC; SEG_F : OUT STD_LOGIC; SEG_G : OUT STD_LOGIC; DP : OUT STD_LOGIC); end SEG_DECODER; architecture Behavioral of SEG_DECODER is signal ONES_BUS : STD_LOGIC_VECTOR(3 downto 0) := "0000"; signal DECS_BUS : STD_LOGIC_VECTOR(3 downto 0) := "0001"; signal HONDREDS_BUS : STD_LOGIC_VECTOR(3 downto 0) := "0000"; begin BIN_TO_BCD : process (ACC_DATA_OUT_BUS) variable hex_src : STD_LOGIC_VECTOR(7 downto 0) ; variable bcd : STD_LOGIC_VECTOR(11 downto 0) ; begin bcd := (others => '0') ; hex_src := ACC_DATA_OUT_BUS; for i in hex_src'range loop if bcd(3 downto 0) > "0100" then bcd(3 downto 0) := bcd(3 downto 0) + "0011" ; end if ; if bcd(7 downto 4) > "0100" then bcd(7 downto 4) := bcd(7 downto 4) + "0011" ; end if ; if bcd(11 downto 8) > "0100" then bcd(11 downto 8) := bcd(11 downto 8) + "0011" ; end if ; bcd := bcd(10 downto 0) & hex_src(hex_src'left) ; -- shift bcd + 1 new entry hex_src := hex_src(hex_src'left - 1 downto hex_src'right) & '0' ; -- shift src + pad with 0 end loop ; HONDREDS_BUS <= bcd (11 downto 8); DECS_BUS <= bcd (7 downto 4); ONES_BUS <= bcd (3 downto 0); end process BIN_TO_BCD; INDICATE : process(CLOCK) type DIGIT_TYPE is (ONES, DECS, HUNDREDS); variable CUR_DIGIT : DIGIT_TYPE := ONES; variable DIGIT_VAL : STD_LOGIC_VECTOR(3 downto 0) := "0000"; variable DIGIT_CTRL : STD_LOGIC_VECTOR(6 downto 0) := "0000000"; variable COMMONS_CTRL : STD_LOGIC_VECTOR(2 downto 0) := "000"; begin if (rising_edge(CLOCK)) then if(RESET = '0') then case CUR_DIGIT is when ONES => DIGIT_VAL := ONES_BUS; CUR_DIGIT := DECS; COMMONS_CTRL := "001"; when DECS => DIGIT_VAL := DECS_BUS; CUR_DIGIT := HUNDREDS; COMMONS_CTRL := "010"; when HUNDREDS => DIGIT_VAL := HONDREDS_BUS; CUR_DIGIT := ONES; COMMONS_CTRL := "100"; when others => DIGIT_VAL := ONES_BUS; CUR_DIGIT := ONES; COMMONS_CTRL := "000"; end case; case DIGIT_VAL is --abcdefg when "0000" => DIGIT_CTRL := "1111110"; when "0001" => DIGIT_CTRL := "0110000"; when "0010" => DIGIT_CTRL := "1101101"; when "0011" => DIGIT_CTRL := "1111001"; when "0100" => DIGIT_CTRL := "0110011"; when "0101" => DIGIT_CTRL := "1011011"; when "0110" => DIGIT_CTRL := "1011111"; when "0111" => DIGIT_CTRL := "1110000"; when "1000" => DIGIT_CTRL := "1111111"; when "1001" => DIGIT_CTRL := "1111011"; when others => DIGIT_CTRL := "0000000"; end case; else DIGIT_VAL := ONES_BUS; CUR_DIGIT := ONES; COMMONS_CTRL := "000"; end if; COMM_ONES <= not COMMONS_CTRL(0); COMM_DECS <= not COMMONS_CTRL(1); COMM_HUNDREDS <= not COMMONS_CTRL(2); SEG_A <= not DIGIT_CTRL(6); SEG_B <= not DIGIT_CTRL(5); SEG_C <= not DIGIT_CTRL(4); SEG_D <= not DIGIT_CTRL(3); SEG_E <= not DIGIT_CTRL(2); SEG_F <= not DIGIT_CTRL(1); SEG_G <= not DIGIT_CTRL(0); DP <= '1'; end if; end process INDICATE; end Behavioral;

Рис. 2 – Часова діаграма ACC

Рис. 3 – Часова діаграма ALU

Рис. 4 – Часова діаграма MUX

Рис. 5 – Часова діаграма RAM

Рис 6. – Часова діграма SEG_DECODER

Рис 7. – Часова діграма TopLevel

Файл TopLevelTest.vhd LIBRARY ieee; USE ieee.std_logic_1164.ALL; USE ieee.numeric_std.ALL; LIBRARY UNISIM; USE UNISIM.Vcomponents.ALL; ENTITY TopLevel_TopLevel_sch_tb IS END TopLevel_TopLevel_sch_tb; ARCHITECTURE behavioral OF TopLevel_TopLevel_sch_tb IS COMPONENT TopLevel PORT( CLOCK : IN STD_LOGIC; RESET : IN STD_LOGIC; ENTER_OP1 : IN STD_LOGIC; ENTER_OP2 : IN STD_LOGIC; CALCULATE : IN STD_LOGIC; DATA_IN : IN STD_LOGIC_VECTOR (7 DOWNTO 0); COMMON_0_OUT : OUT STD_LOGIC; COMMON_1_OUT : OUT STD_LOGIC; COMMON_2_OUT : OUT STD_LOGIC; TEST: OUT STD_LOGIC_VECTOR(7 downto 0); A_OUT : OUT STD_LOGIC; B_OUT : OUT STD_LOGIC; C_OUT : OUT STD_LOGIC; D_OUT : OUT STD_LOGIC; E_OUT : OUT STD_LOGIC; F_OUT : OUT STD_LOGIC; G_OUT : OUT STD_LOGIC; DP_OUT : OUT STD_LOGIC; RAMOUT: OUT STD_LOGIC_VECTOR(7 downto 0); ALUOUT: OUT STD_LOGIC_VECTOR(7 downto 0); RAMA: OUT STD_LOGIC_VECTOR(1 downto 0); RAMWR: OUT STD_LOGIC; OVERFLOW : OUT STD_LOGIC); END COMPONENT; SIGNAL CLOCK : STD_LOGIC := '0'; SIGNAL RESET : STD_LOGIC; SIGNAL ENTER_OP1 : STD_LOGIC; SIGNAL ENTER_OP2 : STD_LOGIC; SIGNAL CALCULATE : STD_LOGIC; SIGNAL DATA_IN : STD_LOGIC_VECTOR (7 DOWNTO 0); SIGNAL COMMON_0_OUT : STD_LOGIC; SIGNAL COMMON_1_OUT : STD_LOGIC; SIGNAL COMMON_2_OUT : STD_LOGIC; SIGNAL A_OUT : STD_LOGIC; SIGNAL B_OUT : STD_LOGIC; SIGNAL C_OUT : STD_LOGIC; SIGNAL D_OUT : STD_LOGIC; SIGNAL E_OUT : STD_LOGIC; SIGNAL F_OUT : STD_LOGIC; SIGNAL G_OUT : STD_LOGIC; SIGNAL DP_OUT : STD_LOGIC; SIGNAL OVERFLOW : STD_LOGIC; SIGNAL TEST: STD_LOGIC_VECTOR(7 downto 0); SIGNAL TEST2: STD_LOGIC_VECTOR(7 downto 0); signal RAMOUT: STD_LOGIC_VECTOR(7 downto 0); signal ALUOUT: STD_LOGIC_VECTOR(7 downto 0); signal RAMA: STD_LOGIC_VECTOR(1 downto 0); signal RAMWR: STD_LOGIC; -- constant CLOCK_period : time := 166ns; constant CLKP: time := 12ms;--24ms; BEGIN UUT: TopLevel PORT MAP( CLOCK => CLOCK, RESET => RESET, ENTER_OP1 => ENTER_OP1, ENTER_OP2 => ENTER_OP2, CALCULATE => CALCULATE, DATA_IN => DATA_IN, COMMON_0_OUT => COMMON_0_OUT, COMMON_1_OUT => COMMON_1_OUT, COMMON_2_OUT => COMMON_2_OUT, A_OUT => A_OUT, B_OUT => B_OUT, C_OUT => C_OUT, D_OUT => D_OUT, E_OUT => E_OUT, F_OUT => F_OUT, G_OUT => G_OUT, DP_OUT => DP_OUT, OVERFLOW => OVERFLOW, TEST => TEST, RAMOUT => RAMOUT, ALUOUT => ALUOUT, RAMA => RAMA, RAMWR => RAMWR ); CLOCK_process: process begin CLOCK <= '0'; wait for 83ns; CLOCK <= '1'; wait for 83ns; end process; -- *** Test Bench - User Defined Section *** tb : PROCESS BEGIN lp1: for i in 4 to 4 loop lp2: for j in 2 to 2 loop TEST2 <= std_logic_vector(unsigned(std_logic_vector(to_unsigned(i + j + 10, 8)) xor std_logic_vector(to_unsigned(j, 8))) - i); ENTER_OP1 <= '1'; ENTER_OP2 <= '1'; CALCULATE <= '1'; DATA_IN <= (others => '0'); RESET <= '0'; wait for CLKP; RESET <= '1'; wait for CLKP; DATA_IN <= std_logic_vector(to_unsigned(i, 8)); -- A ENTER_OP1 <= '0'; wait for CLKP; ENTER_OP1 <= '1'; wait for CLKP; DATA_IN <= std_logic_vector(to_unsigned(j, 8)); -- B ENTER_OP2 <= '0'; wait for CLKP; ENTER_OP2 <= '1'; wait for CLKP; CALCULATE <= '0'; -- START CALCULATION wait for CLKP* 7; assert TEST = TEST2 severity FAILURE; wait for CLKP; end loop; end loop; WAIT; -- will wait forever END PROCESS; -- *** End Test Bench - User Defined Section *** END;

Рис.8 – 7-сегментний індикатор

Переглянемо часову діаграму 7seg decoder.

Бачимо, що для вхідного числа 00010010(2) = 18(10)

Ми отримуємо такі значення(якщо значення не вказане, вважаюмо його за одиницю):

COMM_ONES = 0: B,C,F,G  = 0, що відповідає числу 4
COMM_DECS = 0: B,C = 0, що відповідає числу 1

COMM_HUNDREDS = 0: A,B,C,D,E,F = 0, що відповідає числу 0

Отримуємо значення 014, що збігається з даним йому.

Рис.9 – Успішна прошивка

Висновок: Виконуючи дану лабораторну роботу я навчився реалізовувати цифровий автомат для обчислення значення виразів використовуючи засоби VHDL.